# Diagnostic Patterns by Make and Model

Twenty years of what I saw and what it actually was. Diagnosing by sound what computers miss.

# By Symptom

| Sound or Symptom | Common Cause | Confirm By |
| --- | --- | --- |
| Low growl that changes with steering | Wheel bearing | Spin the wheel off the ground, feel for play |
| Tick at idle, diesel F-250 6.7L | Injector or glow plug | Scan plus cylinder balance test |
| Squeal on cold start, fades | Serpentine belt glazing | Inspect ribs, check tensioner |
| Coolant smell, no visible leak | Heater core or intake gasket | Pressure test, UV dye |
| Clunk over bumps | Sway bar link or strut mount | Lift and pry-bar each joint |

# Rule of Thumb

Trust the ear first, the scanner second.

If two facts conflict, re-test, do not guess a part number.

Write the real cause in the binder so the next mechanic learns it.