Katla Readiness Assessment

Chapter 4: Volcanic Gas Geochemistry and Monitoring Capability

Confidential working draft, v0.7. Prepared for ÍJE management. Not for distribution.

Author:  Martha Glass, Natural Hazards Division, ÍJE

Contributing:  Dr. Helgi Björnsson (glaciology, jökulhlaup scenarios)

Draft date:  18 February 2026

Submission target:  2 October 2026 (to ÍJE management)

4.1  Introduction

This chapter assesses the readiness of gas geochemical monitoring to detect and track unrest at the Katla volcanic system. Katla sits beneath the Mýrdalsjökull ice cap, which complicates direct gas sensing and means that any escalation toward eruption may be accompanied by rapid melting and jökulhlaup. The objective here is narrow and practical: to state what we can currently measure, where the gaps are, and what must be in place so that a change in degassing is recognised early and communicated cleanly to civil protection.

4.2  Current monitoring capability

Gas monitoring of Katla relies on three complementary methods. Ground based DOAS scanning measures sulphur dioxide flux in any plume that clears the ice. Direct sampling with the MultiGAS instrument provides species ratios, principally CO2 over SO2 and H2S over SO2, when fumarolic sites on the caldera rim are accessible. Drone mounted sensors extend reach over crevassed terrain that is unsafe on foot. The instrument fleet is shared with the Reykjanes campaign and the European gas network, so availability during a parallel crisis is a real constraint and is addressed in the recommendations.

| Method | Target species | Strength | Limitation at Katla |
| --- | --- | --- | --- |
| DOAS scanning | SO2 flux | Continuous when plume visible | Ice cap suppresses surface emission |
| MultiGAS | CO2/SO2, H2S/SO2 | Diagnostic of magma input | Needs accessible fumaroles |
| Drone sensors | CO2, SO2 | Reaches unsafe ground | Weather and battery limited |

4.3  Baseline gas measurements and the known gap

Reliable interpretation of any future signal depends on a current baseline. The most complete caldera rim gas survey on file predates the recent acceleration of regional activity, and conditions at the accessible fumarole fields have changed since. This is the single largest gap in our readiness. Without a fresh baseline we risk reading a normal seasonal fluctuation as an anomaly, or worse, dismissing a real change because the reference is stale. A dedicated baseline survey of the caldera rim is the first priority and is scheduled into the summer highland campaign.

4.4  Coupling with jökulhlaup scenarios

Gas escalation and meltwater release are not independent. In coordination with Dr. Helgi Björnsson, this chapter aligns three gas escalation tiers with the corresponding meltwater and jökulhlaup scenarios, so that a single integrated picture reaches decision makers rather than two separate technical notes. The intent is that a measured rise in CO2 over SO2 ratio, paired with subglacial signals, maps to a defined readiness posture.

4.5  Recommendations

First, complete a caldera rim baseline gas survey in the coming field season. Second, secure dedicated instrument capacity so that a Katla response does not compete with Reykjanes for the same units. Third, establish a drone sampling protocol for the ice covered sectors. Fourth, agree the gas to readiness mapping with civil protection in advance, while there is time to discuss it calmly rather than during an event.

This is a scientific working draft. Findings are provisional and are not a hazard forecast. Final assessment and any public guidance rest with ÍJE and the relevant authorities.