TOOLBOX TALK  -  Ladder Safety (5 minutes)

Generic crew safety template. Not job-specific. Not on the books.

Before you climb:

Inspect rails and rungs for cracks or grease.

Set the base one foot out for every four feet of height.

Tie off the top or have someone foot the ladder.

Never stand on the top two rungs.

Face the ladder going up and coming down, three points of contact.

Crew sign-off sheet on the back. File after the Monday stand-up.

Reviewed annually. Nothing here changes for the year-end close.