Finca Miles

Quality Manual | Pressing and Sensory Standards

Estate Quality Manual

Finca Miles, pressing and sensory standards

1. Quality policy

The estate's quality policy is simple: press a smaller harvest well rather than a larger one adequately. Every premium lot must qualify as extra-virgin by chemistry and by an accredited sensory panel, and must trace to its parcel and pressing. This manual records the standards that deliver that.

2. From tree to press

Pick at the target maturity for the lot's intended style (early and green for premium Picual).

Mill within 24 hours of picking; sooner in warm weather.

Keep fruit cool and ventilated between picking and milling.

Clean leaves and debris before milling.

3. Extraction standard

Cold extraction: malaxation temperature held around 24 degrees.

Two-phase decanter; minimal added water.

Press premium, pool and organic-transition fruit in segregated runs.

Settle and rack; light filtration on demand for clarity.

4. Quality limits

| Parameter | EVOO limit | Estate premium target |
| --- | --- | --- |
| Free acidity (%) | <= 0.80 | <= 0.20 |
| Peroxide (meq O2/kg) | <= 20 | <= 8 |
| K270 | <= 0.22 | <= 0.15 |
| Sensory defect | 0 (median) | 0 |
| Sensory panel | - | > 85 / 100 |

5. Sampling and laboratory

Premium lots are sampled and sent to an accredited laboratory for the full panel: free acidity, peroxide value, ultraviolet absorbances (K232, K270, delta-K), ethyl esters, polyphenols and an accredited sensory assessment. The flagship lot MIL-2526-006 this season returned acidity 0.16 percent, peroxide 5.2 and a panel score of 86.5.

6. Certificate of Analysis

Each premium lot's Certificate of Analysis is filed against its lot id and travels with the oil to premium and export buyers. The full per-lot chemistry is maintained in the quality tracker (workbook X08), and a worked certificate is issued as a standalone document (PDF P03).

7. Sensory vocabulary

| Attribute | Meaning |
| --- | --- |
| Fruity | aroma of fresh, sound olives (green or ripe) |
| Bitter | taste of green fruit; a positive in early oil |
| Pungent | peppery throat sensation; high in fresh Picual |
| Defect | fustiness, mustiness, rancidity, none permitted in EVOO |

8. Storage and dispatch

Store oil cool (below 18 C) and dark; keep tanks topped and sealed.

Bottle and tin to order to preserve freshness and polyphenols.

Label premium units with the parcel-and-lot reference.

Dispatch premium and export oil with its Certificate of Analysis.

9. Continuous improvement

Quality results are reviewed lot by lot and season over season. Where the panel or chemistry flags a weakness, a later lot drifting up in acidity, say, the cause is traced to maturity, timing or handling and corrected the following season. The aim is to hold premium sensory scores above 85.

10. Responsibilities

| Role | Quality responsibility |
| --- | --- |
| Min Miles Park | final quality sign-off; pressing and sampling |
| Pedro Serrano | picking timing and fruit handling |
| Soon-Hee Park | traditional tasting input |
| Prof. Maria Castillo (UCO) | pre-harvest condition and pest pressure |

11. Flagship lot, reference results

| Parameter | Result | EVOO limit |
| --- | --- | --- |
| Free acidity (%) | 0.16 | <= 0.80 |
| Peroxide (meq O2/kg) | 5.2 | <= 20 |
| K270 | 0.12 | <= 0.22 |
| Polyphenols (mg/kg) | 612 | (quality) |
| Panel score | 86.5 / 100 | - |

12. Common faults and causes

| Fault | Likely cause | Prevention |
| --- | --- | --- |
| Rising acidity | over-ripe or held fruit | pick on time; mill same day |
| Fusty / musty | fruit piled or fermenting | ventilate; mill within 24 h |
| Rancid | old oil, light, heat, air | store cool and dark; bottle to order |
| Flat / low pungency | late, over-ripe fruit | harvest earlier for premium |

13. Audit of practice

Once a season the owner reviews the pressing and sampling records against this manual, confirms the lab and accreditation are current, and updates the standards where the season's results or new knowledge warrant it.