# Toy Chest, Build Plan

For Sophie, age five. Poplar. Garage workshop. Started October 2026.

# Goal

Goal is a sturdy poplar chest she will still have when she is grown. Soft-close lid so it never catches a small finger. Nothing fancy, just solid and square, the way Grandpa built things.

# Dimensions

Outside, 30 inches wide by 16 inches deep by 17 inches tall to the lid.

Half-inch birch plywood bottom sitting in a groove so it cannot sag.

Three-quarter-inch poplar all around, glued and screwed at the corners.

A post inside each corner for strength and something solid to screw into.

# Order of work

Mill all parts to the cut list, label each piece in pencil.

Cut the bottom groove on the table saw, quarter inch up from the base.

Dry fit the box, check for square corner to corner before any glue.

Glue and clamp the four sides, let it sit overnight.

Glue up the lid panel, flatten with the cleats so it does not cup.

Round every edge with the router so there are no sharp corners.

Sand to 220, then a coat of sanding sealer and two coats of clear poly.

Hang the soft-close hinges and the safety lid stay last.

# Cut list

Rough numbers. Final-fit each joint at the bench, do not cut all four sides to the tape and hope.

| Part | Qty | Material | Size (in) | Notes |
| --- | --- | --- | --- | --- |
| Front and back | 2 | 3/4 poplar | 30 x 15 | Groove for bottom, 1/4 up |
| End panels | 2 | 3/4 poplar | 14-1/2 x 15 | Groove to match |
| Bottom | 1 | 1/2 birch ply | 28-1/2 x 14 | Sits in the groove, floating |
| Corner posts | 4 | 3/4 poplar | 3/4 x 3/4 x 14-1/2 | Glue and screw inside corners |
| Lid panel | 1 | 3/4 poplar | 31 x 17 | Glued up, slight overhang |
| Lid cleats | 2 | 3/4 poplar | 14 long | Screwed on underside, keep it flat |

# Hardware

Soft-close lid hinges so it settles down slow and never slams.

Safety lid stay rated to hold the lid open on its own. This is the whole point, she is only five.

Knob for the front. Sophie picks it at the hardware store, she likes the red ones.

Screws, glue, wood filler for the pilot holes.

# Finishing

Sand through the grits to 220, knock down every edge by hand.

One coat of sanding sealer, let it dry, scuff with a worn 220.

Two coats of clear poly, light sand between coats. Satin, not gloss.

# Notes to self

Let Sophie pick the knob at the hardware store. She likes the red ones.

Keep the safety lid stay. That is the whole point, she is only five.

Burn her initials on the inside of the lid with the wood burner, small, lower corner.